REDWOOD NATIONAL PARK

INVENTORY OF PAST ROAD REHABILITATION SITES

DEFINITIONS FOR FORMS

SITE INFORMATION AND SUMMARY

1.  Rehab Site #: Obtain from Redwood National Park rehabilitation reports.   Usually a 3-digit number with year of rehabilitation listed first:	(Example: 80-3)

2.  Worksite #: Obtain from Redwood National Park rehabilitation maps.  Usually it will be a stream crossing (i.e., Rx 4), road reach (R1) or a landing (L2).

3.  Date mapped: The date of your field work

4.  Mapped by: The initials (first, middle, last) of those who did the field mapping for this particular site (i.e., MAM)

5.  	Watershed: This refers to the major tributary to Redwood Creek in which the inventory site is located (i.e, Bridge Creek).

6.  	Quad ID: Use initials for the appropriate topographic quadrangle, for example:

BH= Bald Hills, RP = Rogers Peak.

7.  Site type: Circle number next to the appropriate item:

Crossing: Locations where a road crossed an ephemeral, intermittent or perennial stream.

Landing: Locations where logs were stored and loaded onto trucks.

Road reach: A length of road that was treated (outsloped, ripped or drained) but without major crossings or landings.

Ditch/Road relief - Locations where a culvert used to drain the inboard ditch, or where waterbars and deep ditches presently drain the old road surface.

Skid trail - sometimes work was done off the main haul road on smaller skid 		trails.

Other - miscellaneous sites such as rock pits.

8.  Erosion Process: Is the erosion at the site caused by running water (fluvial) or a type of landslide (mass movement) or are both types of processes active?

9. Road name - as given by the original timber company, such as the M-7-1 Road.

10:  Year of construction:   Year(s) the road was constructed.  Obtain from Redwood National Park rehabilitation reports or air photos.

11.  Year of rehabilitation: Obtain from Redwood National Park rehabilitation 	reports.

12.  Condition of fill: Describe the condition of the road fill at the site by circlingall the characteristics or features that are present (can circle more than one item.)

Intact: Fill is in good shape.

Sag =sagging.  Has the edge of the road sagged, but no scarps or cracks are visible?  Sagging may mean that scarps or cracks were graded away in the past.

Pond H20 = ponded water.  Are there indications of standing or ponded water at the site?

Cracks: Are there cracks in the road, suggesting initial stage of road fill failue?

Scarps.  Are there scarps in the road with distinct displacement?

Holes.  Holes indicate that fill is falling through the crossing, commonly suggesting the presence of decaying logs within the fill.

Gully/rills.  Are gullies (greater than 1 ft x 1 ft) or rills (less than 1 ft x 1 ft) present on the road surface or on the fill slope?

13. 	. Fill Failure potential: Yes or no.  Does this site have the potential for fill to erode during a large storm (a 20-year storm?)

14.  Rehabilitation Information: (May circle more than one item).

Restoration Technique:

Total or partial outslope (usually refers to road reaches or landings). Total outslope is where the road is recontoured to mimic the natural hillslope, whereas in a partial outslope some road bench remains, and a break in slope between the hillslope and old road surface is obvious.

Total or partial excavation (usually refers to crossings)-- a total excavation removed fill material down to the original channel, and a partial excavation dished out the crossing but did not go as deep.

Ripped: when the road was decompacted by rippers mounted on bulldozers.  It helps increase infiltration on abandoned logging roads.

Drained: Large waterbars or cross road drains were constructed to drain water across the old road surface.

None: Sometimes a segment of road was not treated if it looked stable at the time of rehab.  If such segments are within a treatment area, they get included in this inventory.

15.  Top soil restored?  During total outsloping, the original topsoil that was removed from the road surface during construction is commonly found and replaced on the road surface.  If you can tell by the texture and color of the soil that topsoil was replaced, circle Yes.  If the surface material still looks like road fill, answer No.  If you can’t tell, circle Unknown.

16. Revegetation Treatment: Obtain from Redwood National Park rehabilitation reports.  May circle more than one item.  “Other” may include alder seeding, jute netting, planting of other species, etc.

17-28: For the existing vegetation, estimate an average height and spacing for the given species, if present.  Rank the vegetation types according to dominance with ‘1' as most common, ‘2' the next most common, and so on.  You only need to rank the top 3 or 4 species.  After that, it’s often difficult to decide rank.

29-32: Herbaceous: Basically includes everything that isn’t a tree or a shrub.

Mesic/Xeric will include grasses, forbs, ferns, etc.

Hydrophytic will be those species associated with wet areas and seeps, such as reeds, cattails, horsetails, etc.

Estimate percent cover for herbaceous species: We won’t be able to get too accurate here, but we’re trying to distinguish between heavy cover with almost no soil showing and sparse, with lots of bare soil.

33.  Bedrock type: Get from geology map or field observations.

34.  Soil type: Get from Redwood National Park’s soil maps.

35.  Soil depth: Choose one category based on your field observations in cutbank or streambank exposures.

FLUVIAL EROSION SITE

Always include a sketch on the back of the sheet.

36. Existing erosion feature:

Gully: the site contains a gully as one of the major erosional features.  Gullies are new channels that have a cross-sectional area greater than one square foot.  Anything smaller is considered a rill and is lumped with surface erosion processes.  Gullies are caused by concentrated surface runoff, which often results from stream diversions.

Streambank erosion: The site shows signs of channel widening through erosion of its banks.

Stream incision: The stream has eroded deeper in recent years, usually marked by a distinct break in slope and narrower, incised small channel within a larger channel.

Surface erosion and rilling: Includes rills, sheet erosion, ravelling, soil pedestals, formation of a coarse lag layer on the old road fill surface.

Spring: The crossing area or excavation site drains a spring or seep, which is causing erosion downslope.

Channel Description:

37.  Grade of crossing: Measure longitudinal gradient of excavated crossing with a clinometer.

38, 39.  Do the same for the natural channel upstream and downstream of the excavated area.

40.  Channel width at crossing: What is the estimated width at high flow in the excavated crossing?

41, 42, What is the average channel width upstream and downstream of the excavated crossing?

43: Length of excavated crossing: Measure length from upslope side to downslope side of excavation.

44.  Total drop: This is the elevation difference between the downslope side of the excavation and the upslope side.

45.  Drop due to wood: Frequently logs or other woody debris cause a small waterfall in the channel.  These are sites where much energy is dissipated.  Total the elevation drop for all the wood-based steps in the excavated channels.  For example, two log steps, both 2 ft. high, would yield a total “drop due to wood”of 4 ft.

46.  Drop due to rocks.  Similar to 45, but in this case the channel ‘steps’ are due to boulders or bedrock in the channel bed, causing plunge pools.

47.  Dominant bed material: Circle one that best describes the bed material in the channel bottom.  Sand is less than 2 mm.  P/C = pebbles and cobbles, between 2 mm and 256 mm, Boulders are particles greater than 10 inches median diameter (256 mm).  SmOD is small organic debris (< 6 inches in diameter) and LrgOD is large organic debris (> 6 inches in diameter).

48.  High Bedload.  Subjective assessment if a lot of sand, pebbles and cobbles have been transported through this channel.  If there’s a lot of moss growing on boulders, it’s probably an indication that not much bedload has been transported recently.

49.  Diversion Potential.  Does the site have the potential for flow to be diverted from its natural flow course as a result of conditions at this site?  The most probable conditions for diversion potential are when the channel is not well incised and the old road grade is steep.

50.  Is the stream currently diverted from its natural flow course at this site?

51. Comments: These are useful to explain numbers, and discuss other features that aren’t mentioned on this form.

SECTION II - MASS MOVEMENT SITE

Feature type: Circle those that apply:

Earthflow: An earthflow is a slow moving, deep seated landslide with an irregular and hummocky surface.

Shallow debris slide: A debris slide moves translationally along planar or gently undulating surfaces.  The head scarp is near vertical, and cracks parallel to the slope are usually present in the crown region.  Blocks break up into smaller and smaller parts as the slide moves toward the toe.  Movement is relatively slow as compared to a debris torrent, but  fast in comparison to an earthflow.  If forested, trees will appear jack-strawed or have curved trunks.

Rotational slump: This feature involves movement of a block, or series of blocks, such that displacement is along a concave upward surface.  These features are characterized by steep head scarps, and contain flanks with scarps which decrease in height from the head region to the toe.  The upper surface of the blocks are either flat or tilted back into the hillslope, and may contain trees leaning upslope.  Often the movement grades into a more translational nature toward the lower portion of the slump which may contain a zone of uplift, and trees leaning downslope.

Debris torrent.  This is an extremely rapid downslope movement of material due to complete saturation.  A failed surface contains a serrate or V-shaped scarp, and irregular flanks often with levees in the lower portions.  Displacement occurs along a planar surface, and the surface scar is long and narrow.  Debris torrents typically follow drainage routes, scouring the channel valley to bedrock and mobilizing soil and trees.  They typically build up sufficient energy during failure such that the liquefied material accumulates only at sharp breaks in stream valley slope or orientation.

Cutbank - feature is a failed or slumped cut bank on an old road.

Fill failure - Feature involves perched fill from a road or landing that is failing or has failed downslope.

Failure of excavated fill: This is the case where the road fill material that was excavated moved, and set on the slope (outsloped or put on a fillsite) has subsequently failed since the rehab project was completed.

SLOPE POSITION AND FORM

Circle the responses that define the site’s local position on the hillslope, not its position relative to the entire basin.  It’s helpful to have a topo map with you for this call.

Upper hillslope area.  Is this site within the upper one-third of the slope?

Middle hillslope area.  Is this site within the middle one-third of the slope.

Lower hillslope area.  Is this site within the lower one-third of the slope?

Inner Gorge: Is the site located within the steep side slopes of an inner gorge of a stream channel.  (Usually > 65% slope).

Topographic Form: Circle one.  The general shape of the affected hillslope is best described as:

Concave: Convergent (spoon shaped, or a hollow)

Planar.  Straight

Convex: Divergent, such as the nose of a ridge, watershed divide or interfluve.

BIS.  Break in Slope.  Is the site located at or immediately above a distinct change in hillslope gradient (BIS) which leads from either: moderate slopes above the feature to steeper slopes below, or steeper above and gentler below?

Slope Above (%). The average hillslope gradient immediately upslope of the site.  This question is optional due to the time that may be involved in obtaining a measurement because of brush, etc.

Slope Below (%).  The average gradient immediately downslope of the site.  This question is optional due to the time that may be involved in obtaining a measurement if brush is too thick.

Distance to stream (ft) Indicate the approximate distance to the nearest stream from the toe of the feature.

Comments: Include any other information that you think is pertinent here.

FEATURE DESCRIPTION

Level of Activity: Circle best answer.

Active: Is the site active (movement within the last several years?)  “Active” means the erosion is still occurring, though not necessarily at the original rate.  Gullies will have near vertical, raw banks and/or active headcuts.  Landslides will show recent, mostly bare scarps, recently titled trees and perched blocks which have just started to move.

Waiting: Features assigned this classification are thought to be currently inactive (no signs of movement in the last several years), but the scarps and other indicators suggest that during an especially large storm the instability could become active and fail or move downslope.  This feature type also includes sites which show subtle indicators of future mass movement, but which have not yet moved significantly.

Totally Evacuated.  Has the material associated with the site been completely remove?

Average scarp Height: What is the average scarp height in feet?

Range of scarp heights: What is the range of scarp heights in feet?

Features present: CIRCLE ALL THAT APPLY.

Cracks.  Are there cracks in the road or ground, suggesting slope movement?

Scarps.  Are there scarps in the road or ground with distinct displacement?

Ponded Water: Are there indications of standing or ponded water at the site, if not now, during the wet season?

Sagging.  Has the edge of the road sagged, but no scarps or cracks are visible.  Sagging may mean that scarps or cracks were graded away in the past.

Holes.  Holes indicate that fill is falling through the crossing, often suggesting the presence of decaying logs within the fill.

Leaning trees.  Does the unstable are have leaning or bowed trees?

Spring: Is the mass movement feature a result of emergent ground water?  Cut banks associated with road building occasionally intersect ground water flow, which if not managed properly can lead to erosion.

Stream channel undercutting: Is the site be destabilized (or has the potential for being destabilized) by stream channel undercutting?

Excess water diverted onto feature.  Excess water diverted onto a site can initiate failure and /or accelerate erosion.  Is upslope water diverted to this site?  Is water ponded (in an inboard ditch or poorly drained surface) on the site, causing saturation, which may lead to failure.

SECTION III - TOTAL EROSION VOLUMES

There are four time periods to consider here.

One is how much erosion occurred before the rehabilitation work was done.  You’ll have to read the old reports and study their maps to try to figure this one out, and on some sites you will have no information.  The 1978 air photos may help you in some cases.

Excavated in rehab: This is the volume of material excavated from a stream crossing or removed from a landslide during the actual rehabilitation work.  Again, you’ll need to read the old rehab reports to get a volume excavated, and this information may not always be available.

Erosion since rehab: This is the amount you’ll be measuring in the field based on what you think the ground configuration was after rehab and what has eroded since (in gullies, slumps, incised channels, etc.)

Erosion potential: This is your estimate, based on your field observations, of how much material will eroded during the next 20 years, assuming a large (20-year storm) occurring during this period.  Perched fill, cracks in the fill, undercut or oversteepened banks, are some indicators of potential erosion.  Consider all site conditions and past erosion processes evident within the basin in similar geomorphic, hydrologic, and soil settings in deciding the potential.

The total volume moved (eroded or excavated) is the sum of fluvial-onsite, fluvial-off-site and mass movement features.

Percent delivery to channel.  Estimate the percent of eroded material that entered in the past and will enter in the future to the nearest stream.

Total yield to channel: This is the total volume moved x the delivery percentage.  This is the amount of sediment you think will actually make it to a stream channel in the time periods defined above.

Comments: Add any other comments on the site here.  If you think there is a great potential for a large amount of erosion to occur in the next 20 years at this site, say so and explain why you think you.  Sketches are always helpful.  Use additional paper if necessary.

REDWOOD NATIONAL PARK

INVENTORY OF PAST ROAD REHABILITATION SITES

DEFINITIONS FOR FORMS

SITE INFORMATION AND SUMMARY

1.  Rehab Site #: Get from rehab reports.   Usually a 3-digit number with year of rehab listed first:	(Example: 80-3)

2.  Worksite #: Get from rehab maps.  Usually it will be a stream crossing (i.e., Rx 4), road reach (R1) or a landing (L2).

3.  Date mapped: The date of your field work.

4.  Mapped by: The initials (first, middle, last) of those who did the field mapping for this particular site (i.e., MAM)

5.  Watershed: This refers to the major tributary to Redwood Creek in which the inventory site is located (i.e, Bridge Creek).

6.  Quad ID: Use initials for the appropriate topographic quad:

BH= Bald Hills, RP = Rogers Peak.

7.  Site type: Circle number next to the appropriate item:

Crossing: Locations where a road crossed an ephemeral, intermittent or perennial stream.

Landing: Locations where logs were stored and loaded onto trucks.

Road reach: A length of road that was treated (outsloped, ripped or drained) but without major crossings or landings.

Ditch/Road relief - Locations where a culvert used to drain the inboard ditch, or where waterbars and deep ditches presently drain the old road surface.

Skid trail - sometimes work was done off the main haul road on smaller skid trails.

Other - miscellaneous sites such as rock pits.

8.  Erosion Process: Is the erosion at the site caused by running water (fluvial) or a type of landslide (mass movement) or are both types of processes active?

9. Road name - as given by the timber company, such as the M-7-1 Rd.

10:  Year of construction:   Year(s) the road was constructed.  Get from rehab reports or air photos.

11.  Year of rehab: Get from rehab reports.

12.  Condition of fill: Describe the coniditon of the road fill at the site by circlingall the characteristics or features that are present (can circle more than one item.)

Intact: Fill is in good shape.

Sag =sagging.  Has the edge of the road sagged, but no scarps or cracks are visible?  Sagging may mean that scarps or cracks were graded away in the past.

Pond H20 = ponded water.  Are there indications of standing or ponded water at the site?

Cracks: Are there cracks in the road, suggesting initial stage of road fill failue?

Scarps.  Are there scarps in the road with distinct displacement?

Holes.  Holes indicate that fill is falling through the crossing, commonly suggesting the presence of decaying 	logs within the fill.

Gully/rills.  Are gullies (greater than 1 ft x 1 ft) or rills (less than 1 ft x 1 ft) present on the road surface or on the fill slope?

13. Fill Failure potential: Yes or no.  Does this site have the potential for fill to erode during a large storm (say a 20-year storm?)

14.  Rehabilitation Info: (May circle more than one item).

Restoration Technique: Total or partial outslope (usually refers to road reaches or landings). Total outslope is where the road is recontoured to mimic the natural hillslope, whereas in a partial outslope some road bench remains, and a break in slope between the hillslope and old road surface is obvious.

Total or partial excavation (usually refers to crossings)-- a total excavation removed fill material down to the original channel, and a partial excavation dished out the crossing but did not go as deep.

Ripped: when the road was decompacted by rippers mounted on bulldozers.  It helps increase infiltration on 	abandoned logging roads.

Drained: Large waterbars or cross road drains were constructed to drain water across the old road surface.

None: Sometimes a segment of road was not treated if it looked stable at the time of rehab.  If such segments are now eroding, they get included in this inventory.

15.  Top soil restored?  During total outsloping, the original topsoil that was removed from the road surface during construction is commonly found and replaced on the road surface.  If you can tell by the texture and color of the soil that topsoil was replaced, circle Yes.  If the surface material still looks like road fill, answer No.  If you can’t tell, circle Unknown.

16 Revegetation Treatment: Need to get from rehab reports.  May circle more than one item.  “Other” may include alder seeding, jute netting, planting of other species, etc.

17-28: For the existing vegetation, estimate an average height and spacing for the given species, if present.  Rank the vegetation types according to dominance with ‘1' as most common, ‘2' the next most common, and so on.  You only need to rank the top 3 or 4 species.  After that, it’s often difficult to decide rank.

29-32     .Herbaceous: Basically includes everything that isn’t a tree or a shrub.

Mesic/Xeric will include grasses, forbs, ferns, etc.

Hydrophytic will be those species associated with wet areas and seeps, such as reeds, cattails, horsetails, etc.

Estimate percent cover for herbaceous species: We won’t be able to get too accurate here, but we’re trying to distinguish between heavy cover with almost no soil showing and sparse, with lots of bare soil.

33.  Bedrock type: Get from geology map or field observations.

34.  Soil type: Get from Jim Popenoe’s soil maps.

35.  Soil depth: Choose one category based on your field observations in cutbank or streambank exposures.

FLUVIAL EROSION SITE

Always include a sketch on the back of the sheet.

36. Existing erosion feature:

Gully: the site contains a gully as one of the major erosional features.  Gullies are new channels that have a cross-sectional area greater than one square foot.  Anything smaller is considered a rill and is lumped with surface erosion processes.  Gullies are caused by concentrated surface runoff, which often results from stream diversions.

Streambank erosion: The site shows signs of channel widening through erosion of its banks.

Stream incision: The stream has eroded deeper in recent years, usually marked by a distinct break in slope and narrower, incised small channel within a larger channel.

Surface erosion and rilling: Includes rills, sheet erosion, ravelling, soil pedestals, formation of a coarse lag layer on the old road fill surface.

Spring: The crossing area or excavation site drains a spring or seep, which is causing erosion downslope.

Channel Description:

37.  Grade of crossing: Measure longitudinal gradient of excavated crossing with a clinometer.

38, 39.  Do the same for the natural channel upstream and downstream of the excavated area.

40.  Channel width at crossing: What is the estimated width at high flow in the excavated crossing?

41, 42, What is the average channel width upstream and downstream of the excavated crossing?

43: Length of excavated crossing: Measure length from upslope side to downslope side of excavation.

44.  Total drop: This is the elevation difference between the downslope side of the excavation and the upslope side.

45.  Drop due to wood: Frequently logs or other woody debris cause a small waterfall in the channel.  These are sites where much energy is dissipated.  Total the elevation drop for all the wood-based steps in the excavated channels.  For example, two log steps, both 2 ft. high, would yield a total “drop due to wood”of 4 ft.

46.  Drop due to rocks.  Similar to 45, but in this case the channel ‘steps’ are due to boulders or bedrock in the channel bed, causing plunge pools.

47.  Dominant bed material: Circle one that best describes the bed material in the channel bottom.  Sand is less than 2 mm.  P/C = pebbles and cobbles, between 2 mm and 256 mm, Boulders are particles greater than 10 inches median diameter (256 mm).  SmOD is small organic debris (< 6 inches in diameter) and LrgOD is large organic debris (> 6 inches in diameter).

48.  High Bedload.  Subjective assessment if a lot of sand, pebbles and cobbles have been transported through this channel.  If there’s a lot of moss growing on boulders, it’s probably an indication that not much bedload has been transported recently.

49.  Diversion Potential.  Does the site have the potential for flow to be diverted from its natural flow course as a result of conditions at this site?  The most probable conditions for diversion potential are when the channel is not well incised and the old road grade is steep.

50.  Is the stream currently diverted from its natural flow course at this site?

51. Comments: These are useful to explain numbers, and discuss other features that aren’t mentioned on this form.

SECTION II - MASS MOVEMENT SITE

Feature type: Circle those that apply:

Earthflow: An earthflow is a slow moving, deep seated landslide with an irregular and hummocky surface.

Shallow debris slide: A debris slide moves translationally along planar or gently undulating surfaces.  The head scarp is near vertical, and cracks parallel to the slope are usually present in the crown region.  Blocks break up into smaller and smaller parts as the slide moves toward the toe.  Movement is relatively slow as compared to a debris torrent, but  fast in comparison to an earthflow.  If forested, trees will appear jack-strawed or have curved trunks.

Rotational slump: This feature involves movement of a block, or series of blocks, such that displacement is along a concave upward surface.  These features are characterized by steep head scarps, and contain flanks with scarps which decrease in height from the head region to the toe.  The upper surface of the blocks are either flat or tilted back into the hillslope, and may contain trees leaning upslope.  Often the movement grades into a more translational nature toward the lower portion of the slump which may contain a ne of uplift, and trees leaning downslope.

Debris torrent.  This is an extremely rapid downslope movement of material due to complete saturation.  A failed surface contains a serrate of V-shaped scarp, and irregular flanks often with levees in the lower portions.  Displacement occurs along a planar surface, and the surface scar is long and narrow.  Debris torrents typically follow drainage routes, scouring the channel valley to bedrock and mobilizing soil and trees.  They typically build up sufficient energy during failure such that the liquefied material accumulates only at sharp breaks in stream valley slope or orientation.

Cutbank - feature is a failed or slumped cut bank on an old road.

Fill failure - Feature involves perched fill from a road or landing that is failing or has failed downslope.

Failure of excavated fill: This is the case where the road fill material that was excavated moved, and set on the slope (outsloped or put on a fillsite) has subsequently failed since the rehab project was completed.

SLOPE POSITION AND FORM

Circle the responses that define the site’s local position on the hillslope, not its position relative to the entire basin.  It’s helpful to have a topo map with you for this call.

Upper hillslope area.  Is this site within the upper one-third of the slope?

Middle hillslope area.  Is this site within the middle one-third of the slope.

Lower hillslope area.  Is this site within the lower one-third of the slope?

Inner Gorge: Is the site located within the steep side slopes of an inner gorge of a stream channel.  (Usually > 70% slope).

Topographic Form: Circle one.  The general shape of the affected hillslope is best described as:

Concave: Convergent (spoon shaped, or a hollow)

Planar.  Straight

Convex: Divergent, such as the nose of a ridge, watershed divide or interfluve.

BIS.  Break in Slope.  Is the site located at or immediately above a distinct change in hillslope gradient (BIS) which leads from either: moderate slopes above the feature to steeper slopes below, or steeper above and gentler below?

Slope Above (%). The average hillslope gradient immediately upslope of the site.  This question is optional due to the time that may be involved in obtaining a measurement because of brush, etc.

Slope Below (%).  The average gradient immediately downslope of the site.  This question is optional due to the time that may be involved in obtaining a measurement if brush is too thick.

Distance to stream (ft) Indicate the approximate distance to the nearest stream from the toe of the feature.

Comments: Include any other information that you think is pertinent here.

FEATURE DESCRIPTION

Level of Activity: Circle best answer.

Active: Is the site active (movement within the last several years?)  “Active” means the erosion is still occurring, though not necessarily at the original rate.  Gullies will have near vertical, raw banks and/or active headcuts.  Landslides will show recent, mostly bare scarps, recently titled trees and perched blocks which have just started to move.

Waiting: Features assigned this classification are thought to be currently inactive (no signs of movement in the last several years), but the scarps and other indicators suggest that during an especially large storm the instability could become active and fail or move downslope.  This feature type also includes sites which show subtle indicators of future mass movement, but which have not yet moved significantly.

Totally Evacuated.  Has the material associated with the site been completely remove?

Average scarp Height: What is the average scarp height in feet?

Range of scarp heights: What is the range of scarp heights in feet?

Features present: CIRCLE ALL THAT APPLY.

Cracks.  Are there cracks in the road or ground, suggesting slope movement?

Scarps.  Are there scarps in the road or ground with distinct displacement?

Ponded Water: Are there indications of standing or ponded water at the site, if not now, during the wet season?

Sagging.  Has the edge of the road sagged, but no scarps or cracks are visible.  Sagging may mean that scarps or cracks were graded away in the past.

Holes.  Holes indicate that fill is falling through the crossing, often suggesting the presence of decaying logs within the fill.

Leaning trees.  Does the unstable are have leaning or bowed trees?

Spring: Is the mass movement feature a result of emergent ground water?  Cut banks associated with road building occasionally intersect ground water flow, which if not managed properly can lead to erosion.

Stream channel undercutting: Is the site be destabilized (or has the potential for being destabilized) by stream channel undercutting?

Excess water diverted onto feature.  Excess water diverted onto a site can initiate failure and /or accelerate erosion.  Is upslope water diverted to this site?  Is water ponded (in an inboard ditch or poorly drained surface) on the site, causing saturation, which may lead to failure.

SECTION III - TOTAL EROSION VOLUMES

There are four time periods to consider here.

One is how much erosion occurred before the rehabilitation work was done.  You’ll have to read the old reports and study their maps to try to figure this one out, and on some sites you will have no information.  The 1978 air photos may help you in some cases.

Excavated in rehab: This is the volume of material excavated from a stream crossing or removed from a landslide during the actual rehabilitation work.  Again, you’ll need to read the old rehab reports to get a volume excavated, and this information may not always be available.

Erosion since rehab: This is the amount you’ll be measuring in the field based on what you think the ground configuration was after rehab and what has eroded since (in gullies, slumps, incised channels, etc.)

Erosion potential: This is your estimate, based on your field observations, of how much material will eroded during the next 20 years, assuming a large (20-year storm) occurring during this period.  Perched fill, cracks in the fill, undercut or oversteepened banks, are some indicators of potential erosion.  Consider all site conditions and past erosion processes evident within the basin in similar geomorphic, hydrologic, and soil settings in deciding the potential.

The total volume moved (eroded or excavated) is the sum of fluvial-onsite, fluvial-off-site and mass movement features.

Percent delivery to channel.  Estimate the percent of eroded material that entered in the past and will enter in the future to the nearest stream.

Total yield to channel: This is the total volume moved x the delivery percentage.  This is the amount of sediment you think will actually make it to a stream channel in the time periods defined above.

Comments: Add any other comments on the site here.  If you think there is a great potential for a large amount of erosion to occur in the next 20 years at this site, say so and explain why you think you.  Sketches are always helpful.  Use additional paper if necessary.